Osprey DOS File Access Device

PRELIMINARY

Version 1.1 -- April 14, 1994

Introduction

This document describes the low level mechanism by which programs running on the Osprey can access files in the host (DOS for now) system's directory structure.

Access to host files is accomplished through a "hardware" device which performs the various file access functions.

For now all data transfers are in binary--any record handling or line terminator fixup must be done by the PDP program.  The PDP program is also responsible for dealing with problems resulting from DOS's 8.3 filename limitation.  As other host environments are supported, commands will be added to allow the PDP program to determine the filename formats supported by the host.

The device is enabled by adding a line to the Osprey configuration file:

DosDevice	=	DosDevice [Options]

The following table lists the supported options:

| Option | Function | Legal range | Default |
| --- | --- | --- | --- |
| /Adr:<Octal> | Set device CSR address | 160000-177777 | 176470 |
| /Vct:<Octal> | Set device interrupt vector | 000-774 | 240 |
| /Pri:<Decimal> | Set device interrupt priority | 4-7 | 4 |

The device interrupt vector can also be programmed at run time through the CSR register.

To control the DSF device, PDP program sets up command and data buffers in memory, then initiates the command by passing the command buffer address to the device and setting the START bit in the device's CSR register.  The remainder of this document defines the device registers and command buffer layout.

DOS File Device

All commands to the DSF device follow the same sequence:

1)	Fill in the command definition block

2)	Fill in the command data block if required

3)	Give the address of command definition block to the device

4)	Set the START bit the CSR to start the command

5)	Wait for the command to complete by polling the CSR or waiting for device to interrupt

6)	Check the command block status word for possible errors

Dos File Device Register Definitions

| Offset | Name | Function |
| --- | --- | --- |
| 0 | CSR | Control/Status Register |
| 2 | LADR | Command Block Address, low order |
| 4 | HADR | Command Block Address, bits 0-5 only, others always zero |

DSF CSR Bit Definitions

| Bit | Read/Write | Function |
| --- | --- | --- |
| 0 | Read/Write | Enables device interrupt upon command completion |
| 1 | Write | START bit--set to 1 by driver to initiate a command |
| 1 | Read | Indicates device is processing current command, cleared when done |
| 2 | Read only | Indicates device has completed current command, cleared on read |
| 3 | Read only | Unused, read as 0 |
| 4-7 | Read only | Device priority level |
| 8-14 | Read/Write | Device interrupt vector / 4 |
| 15 | Read Only | Unused, read as 0 |

DSF Command Block Format

| Word | Function |
| --- | --- |
| 0 | Command code |
| 2 | Command status (set by device) |
| 4 | File Number |
| 6 | Optional command parameter |
| 10 | Data buffer length in bytes |
| 12 | Data buffer address, low order |
| 14 | Data buffer address, high order |

DSF Command Definitions

Command code 0 -- Open DOS File

File Number:		Returned by Host on successful file open

Parameter:		File open mode, see below

Buffer Addr:		Points to zero terminated Host file name to be opened

Buffer Size:		Length of filename, including terminating zero (max 128 bytes)

The device will read in the filename, and attempt to open the file in the appropriate mode.  Data buffer may contain any valid Host filename.  Upon success, the device will return a file number to be used to reference the file in later commands such read and write data.

The file open mode parameter contains several fields:

DSF Open command parameter word fields

| Bit | Function |
| --- | --- |
| 0-1 | Requested access mode |
| 2-4 | DOS Sharing mode |

The following octal field values are supported:

Read/Write Access Mode:	READ_WRITE	0000

READ_ONLY	0001

WRITE_ONLY	0002

Sharing Mode:		COMPATIBILITY	0000

DENY_NONE	0004

DENY_READ	0010

DENY_WRITE	0014

EXCLUSIVE		0020

For now, all data transferred is strictly binary byte-stream; no data formatting is performed by the host.  It the future, the open-mode parameter my be also contain re-formatting information.

Command code 1 -- Create DOS File

File Number:		Unused

Parameter:		Unused

Buffer Addr:		Points to zero terminated Host file name to be created

Buffer Size:		Length of filename, including terminating zero (max 128 bytes)

Command code 2 -- Delete DOS File

File Number:		Unused

Parameter:		Unused

Buffer Addr:		Points to zero terminated Host file name to be deleted

Buffer Size:		Length of filename, including terminating zero (max 128 bytes)

Command code 3 -- Read DOS File

File Number:		As returned by the OPEN command

Parameter:		Unused

Buffer Addr:		Address of area to receive the file data

Buffer Size:		Number of bytes requested.  Host returns number of bytes actually read; will never be more than requested length.  On end of file error status, a the read count returned will be less than the requested count.

Command code 4 -- Write DOS File

File Number:		As returned by the OPEN command

Parameter:		Unused

Buffer Addr:		Address of actual file data buffer

Buffer Size:		Number of bytes to write.  Host returns number of bytes actually written.

Command code 5 -- Set File Position

File Number:		As returned by the OPEN command

Parameter:		Position relative to: 0 = Front, 1 = Current position, 2 = End

Buffer Addr:		32 bit signed number.  Host returns actual file position

Buffer Size:		Unused

Command code 6 -- Find first matching file

File Number:		Unused

Parameter:		Unused

Buffer Addr:		Points to zero terminated host file name template

Buffer Size:		Length of file name buffer (max 128 bytes)

Command code 7 -- Find next matching file

File Number:		Unused

Parameter:		Unused

Buffer Addr:		Points to buffer to receive next filename

Buffer Size:		Length of file name buffer (max 128 bytes)

These commands allow scanning of a host directory.  The host returns the first/next filename from the directory which matches the input pattern.  Only one "first,next,...,next" sequence is valid at a time--i.e. it is not possible to intermix scanning of two host directories.

Command code 8 -- Close DOS file

File Number:		As returned by the OPEN command

Parameter:		Unused

Buffer Addr:		Unused

Buffer Size:		Unused

After each command, the PDP driver code should check the status word to make sure the command completed correctly.  The following table defines the current error codes possible from the DSF device:

DSF Device Status Codes

| Value | Function |
| --- | --- |
| 0 | No error--command was successful |
| 1 | End of file encountered on read |
| 2 | File or Directory does not exist |
| 9 | Invalid file number |
| 12 | Not enough memory |
| 13 | Permission denied |
| 17 | File already exists |
| 18 | Cross device link |
| 22 | Invalid command or argument |
| 24 | Too many open files |
| 28 | No space left on device |
| 36 | Resource deadlock would occur |